MATERIAL SAFETY DATA SHEET

    - CHEMICAL IDENTITY

- Chemical Name : High Speed Diesel				Chemical classification : Fuel

- Synonyms          : Gas Oil				              	Trade Name                  : Diesel Oil

Formula : A Complex Mixture of hydrocarbons            	C.A.S. No.             		   	U.N.No. : 1202

- Regulated Identification     Shipping Name : Diesel oil				    Hazchem No.                     : 3 Z

- Codes / Label    : 30/Class 3, Flammable Liquid                    Hazardous Waste I.D No. : 5

- HAZARDOUS INGREDIENTS                   		C.A.S. No.

2.    PHYSICAL AND CHEMICAL DATA

Boiling Range  : 110 to 400oC	Physical state    : Liquid @ 15oC & 1 atm	Appearance: yellow to oily brown

- Vapour pressure: < 0.1 psi @ 38oC		Odour         : Characteristic

Freezing point: 0 - 18°C		Others: Insoluble and floats in water

- Vapour Density: 3.0 to 5.0 (Air = 1)

- Specific Gravity: 0.840 gm/ml  @ 15oC					pH: Not pertinent

- 3.    FIRE AND EXPLOSION  HAZARD DATA

- Flammability           : Yes			LEL  : 0.5 %		Flash Point  : > 32°C min		(CC)

- TDG Flammability  : 3			UEL  : 5.0 %		Flash Point  : > 35°C min		(OC)

- Auto ignition Temperature  	                   : 230 - 300oC

- Explosion sensitivity to impact 	   	  : Stable

- Explosion sensitivity to static Electricity           :  Stable

- Hazardous Combustion Products 	   	  : Acrid smoke and irritating fumes of CO / CO2 / NOX

- Hazardous Polymerization 		  : Does not occur

- Combustible Liquid  : Yes	Explosive Material : No		Corrosive Material : No

- Flammable Material : Yes		Oxidiser                 : No		Others                    : No

- Pyropheric Material  : No 		Organic Peroxide   : No

- 4.    REACTIVITY DATA

- Chemical Stability			: Chemically Stable

- Incompatibility with other material		: Incompatible with strong oxidisers.

- Reactivity				: Does not react with common materials but may react with oxidising agents.

- Hazardous Reaction products		: Data not available

- 5.    HEALTH HAZARD DATA

- Routes of entry	: Inhalation / Ingestion / Skin / Eye

- Effects of Exposure /	: Inhalation : Dizziness, headache

- Symptoms		                       and bluish tint to the skin.

- : Ingestion   : Nausea and vomiting. Irritation of mouth and gastro intestinal tract may follow, rapidly developing potentially fatal chemical

- Skin          :   Irritation, will remove fat from the skin. Prolonged or repeated contact should be avoided, otherwise skin chapping, cracking or possible contact dermatitis may occur. Dry skin, erythema, oil acne, and oil folliculitis & watery growth may occur which may become skin cancer subsequently on repeated exposure.

- Eye           :  eye irritation

- Emergency Treatment : If inhaled, Remove victim to fresh air, give artificial respiration if required, remove contaminated clothing quickly, do not induce vomiting if ingested, keep victim warm and quiet.  Wash all the affected skin thoroughly with soap and water. Irrigate affected eyes with copious amount of water. Administration of  Olive oil or any other vegetable oil may reduce adsorption thru digestive tract. Gastric lavage should be done only after endotracheal intubation in view of respiration which may cause serious chemical pneumonitis for which antibiotic and corticosteroid therapy may be indicated.

- TLV ( ACGIH)           : Not listed					STEL 	               : Not listed

- PEL                            : Not listed				               Odour Threshold    : Not listed

- LD50                           :  0.5 to 5 gm/Kg				Limit LD50             : Not listed

- NFPA Hazard signal	Health : 1		Flammability : 3		Reactivity : 0     		Special : Nil

- 6.   PREVENTIVE MEASURES

- Personnel Protective Equipment	: Avoid contact with liquid or vapour.

- Use gum boots, gloves while handling the product.

Handling & Storage Precautions	: Store in dry, cool, ventilated area away from heat and flame.

Keep away from oxidising agents

7.   EMERGENCY AND FIRST AID MEASURES

FIRE		Fire Extinguishing Media	: Foam / DCP / CO2.

Special Procedures		: Keep the containers cool by spraying water if exposed to fire.

Unusual Hazards		: Flashback may occur along vapour trail.

EXPOSURE	First Aid Measures	: A/a		Antidotes / Dosages : Not available

SPILLS	Steps to be Taken		: Shut off leaks without risk.

Contain leaking liquid on sand or earth

Prevent liquid from entering into sewers.

Waste Disposal Method	: Seal all the waste in vapour tight plastic bags for eventual disposal.

8.   ADDITIONAL INFORMATION / REFERENCES

9.   MANUFACTURERS / SUPPLIERS DATA   : HINDUSTAN PETROLEUM CORPORATION LIMITED

10. DISCLAIMER

Information contained in this material data sheet is believed to be reliable but no representation, guarantee or warranties of any kind are made as to its accuracy, suitability for a particular application or results to be obtained from them.  It is upto the end user to ensure that information contained in the material safety data sheet is relevant to the product manufactured / handled or sold by him as the case may be and H.P.C.L. makes no warranties expressed or implied in respect of adequacy of this document for any particular purpose.

MATERIAL SAFETY DATA SHEET

    - CHEMICAL IDENTITY

- Chemical Name  : Motor Spirit					Chemical classification : Fuel

- Synonyms          : Gasoline				              	Trade Name                  : Petrol

Formula : A Complex Mixture of hydrocarbons            	C.A.S. No. 8006-61-9	   	U.N.No. : 1203

- Regulated Identification     Shipping Name : Gasoline				    Hazchem No.                     : 3 Y E

- Codes / Label    : 33/Class 3, Flammable Liquid                    Hazardous Waste I.D No. : 5

- HAZARDOUS INGREDIENTS                   		C.A.S. No.

- Motor Spirit					8006-61-9

2.    PHYSICAL AND CHEMICAL DATA

Boiling Range  : 30°C  to 215oC	           Physical state    : Liquid @ 15oC & 1 atm	     Appearance: Orange (Octane 87)

Red (Octane 93)

- Vapour pressure: @ 35oC		      Odour         : Aromatic Odour

Freezing point: - 40°C			 : 6 to 10psi @ 38oC 		      Others        : Easily soluble in 												           Alcohol, Ether, 												          Chloroform &  Benzene

- Vapour Density: 3.0 to 4.0 (Air = 1)          Solubility in water @ 30°C

- : Insoluble in water

- Floats on water

- Specific Gravity: 0.733 gm/l  @ 15.6oC      pH  (Water = 1)	: Not pertinent

- 3.    FIRE AND EXPLOSION  HAZARD DATA

- Flammability           : Yes			LEL  : 1.4 %		Flash Point  : - 30°C min		(CC)

- TDG Flammability  : 3			UEL  : 7.6 %		Flash Point  : Data not available	(OC)

- Auto ignition Temperature  	                   : 390oC

- Explosion sensitivity to impact 	   	  : Stable

- Explosion sensitivity to static Electricity           :  May explode

- Hazardous Combustion Products 	   	  : CO / CO2 / SO2 / NOX

- Hazardous Polymerization 		  : Does not occur

- Combustible Liquid  : Yes	Explosive Material : No		Corrosive Material : No

- Flammable Material : Yes		Oxidiser                 : No		Others                    : No

- Pyropheric Material  : No 		Organic Peroxide   : No

- 4.    REACTIVITY DATA

- Chemical Stability			: Chemically Stable

- Incompatibility with other material		: Incompatible with strong oxidisers.

- Reactivity				: Does not react with common materials but may react with oxidising agents.

- Hazardous Reaction products		: Data not available

- 5.    HEALTH HAZARD DATA

- Routes of entry	: Inhalation / Ingestion / Dermal absorption / Eye

- Effects of Exposure /    : Inhalation : Inhalation in very high concentration of vapour causes loss of consciousness, coma               Symptoms                                         and sudden death.  In  less severe cases causes headache, nausea and mental                                                                                      confusion and depression may occur

- : Ingestion  : Ingestion causes irritation of gastro intestinal tract with vomiting colic and diarrhea

- : Skin           :  Prolonged skin contact will dry an defat skin with dermatitis

- : Eye            :  Contact with eye causes pain and slight transient corneal ephithelial disturbaxes

- Emergency Treatment :   If drenched with product remove soaked clothings immediately.  Flush afffected area with plenty of water and then was with soap and water.  If  inhaled, remove the victim to fresh air.  If not breathing, give artificial respiration.  If breathing is difficult, call physician.  If swallowed, do not induce vomiting.  If eye is affected, flush with plenty of water until irritation subsides.  Keep patient warm and lying down.  Give hot drinks like milky tea or coffee.  Call physician or transport vicitim to hospital, if necessary.  If  ingested, liquid paraffin, olive oil or any other vegetable oil is to be given orally to retard absorption of gasoline.  Gastric lavage and the induction of vomiting are not advisable beacuse of possibility of aspiration of gasoline.  Gastric lavage  and the induction of vomiting are not advisable because of possibility of aspirationof gasoline and the subsequent development of chemical pneumenia.

- TLV ( ACGIH)          :  300 ppm             900 mg/m3  		STEL 	                :  500  ppm         1500 mg/m3

- PEL                            :  300 ppm	      900 mg/m3		Odour Threshold    : 0.25 ppm           300 mg/m3

- LD50                           :   Not Listed                                                     LD50                         :   Not Listed

- NFPA Hazard Signals	Health : 1		Flammability : 4		Reactivity : 0     		Special : Nil

- 6.   PREVENTIVE MEASURES

- Personnel Protective Equipment		: Hand gloves / Gas mask / PVC suit.

Handling & Storage Precautions	:

Avoid contact with liquid or Vapours 			Use flameproof electrical equipment only

Stay upwind while gauging / Sampling / handling	Earth all equipment and pipelines properly.

Do not wash / clean hands with the product		Store in an enclosed vessel in a cool, well ventilated area 			Do not suck the product				from heat & flame

Gas free the tank before entering / cleaning		No smoking or open flames

Change oil soaked clothing promptly			Provide adequate ventilation at work site

7.   EMERGENCY AND FIRST AID MEASURES

FIRE		Fire Extinguishing Media	: Foam / DCP / CO2.

Special Procedures		: Keep the containers cool by spraying water if exposed to fire.

Unusual Hazards		: Flashback may occur along vapour trail.

EXPOSURE	First Aid Measures	: A/a		Antidotes / Dosages : Not available

SPILLS	Steps to be Taken		: Eliminate all sources of ignition.

Ventilate the area.

Stop leaks if no risk involved

Collect leaking product into closed container.

Contain / absorb spillage in sand / earth bund.

Use water spray to disperse / dilute the vapours, if necessary

Prevent run-off from entering into sewers.

Waste Disposal Method	: Colelct all the waste in vapour tight plastic bags for eventual disposal.

8.   ADDITIONAL INFORMATION / REFERENCES

9.   MANUFACTURERS / SUPPLIERS DATA   : HINDUSTAN PETROLEUM CORPORATION LIMITED

10. DISCLAIMER

Information contained in this material data sheet is believed to be reliable but no representation, guarantee or warranties of any kind are made as to its accuracy, suitability for a particular application or results to be obtained from them.  It is upto the end user to ensure that information contained in the material safety data sheet is relevant to the product manufactured / handled or sold by him as the case may be and H.P.C.L. makes no warranties expressed or implied in respect of adequacy of this document for any particular purpose.